In a basic computer , the instruction code is stored in:
A) DR
B) AC
C) IR
D) TR

✅Answer: C) IR

Q2. In a 16-bit instruction format, if 4 bits are used for opcode, maximum number of instructions possible are:
A) 8
B) 16
C) 32
D) 64

✅Answer: B) 16

Q3. In indirect addressing mode, the effective address is obtained from:
A) Operand field directly
B) Memory location specified in instruction
C) PC
D) AC

✅Answer: B) Memory location specified in instruction

Q4. The I bit in instruction format of basic computer indicates:
A) Interrupt
B) Indirect addressing
C) Immediate data
D) I/O operation

✅Answer: B) Indirect addressing

🔸 2. Computer Registers

Q5. Which register holds the address of next instruction?
A) IR
B) AR
C) PC
D) DR

✅Answer: C) PC

Q6. The register that acts as memory buffer register is:
A) DR
B) TR
C) AR
D) AC

✅Answer: A) DR

Q7. Stack pointer (SP) in a stack organization always points to:
A) Bottom of stack
B) Next free location
C) Top of stack
D) Instruction

✅Answer: C) Top of stack

🔸 3. Common Bus System

Q8. In a common bus system using multiplexers, the selection lines are equal to:
A) log₂(number of registers)
B) number of registers
C) number of bits
D) number of control signals

✅Answer: A) log₂(number of registers)

Q9. The common bus system reduces:
A) Memory size
B) Number of registers
C) Hardware complexity of connections
D) Instruction size

✅Answer: C) Hardware complexity of connections

Q10. In a 4-register bus system, number of selection lines required is:
A) 1
B) 2
C) 3
D) 4

✅Answer: B) 2

🔸 4. Computer Instructions

Q11. A three-address instruction is of the form:
A) X = Y op Z
B) X op Y
C) op X
D) X = op

✅Answer: A) X = Y op Z

Q12. Which instruction does not reference memory?
A) AND
B) ADD
C) CLA
D) LDA

✅Answer: C) CLA

Q13. Register reference instructions are identified when:
A) Opcode = 111 and I = 0
B) Opcode = 111 and I = 1
C) Opcode = 000
D) I = 1

✅Answer: A) Opcode = 111 and I = 0

🔸 5. Timing and Control

Q14. Sequence counter is used to generate:
A) Clock pulses
B) Timing signals T0, T1, T2…
C) Interrupt signals
D) Address signals

✅Answer: B) Timing signals T0, T1, T2…

Q15. The fetch cycle of basic computer takes how many timing signals?
A) 1
B) 2
C) 3
D) 4

✅Answer: C) 3

Q16. The instruction cycle consists of:
A) Fetch only
B) Fetch + Execute
C) Decode only
D) Interrupt only

✅Answer: B) Fetch + Execute

🔸 6. Instruction Cycle

Q17. During fetch cycle:
A) IR ← M[AR]
B) DR ← AC
C) AC ← DR
D) PC ← AC

✅Answer: A) IR ← M[AR]

Q18. If PC = 200 and instruction length is 1 word, after fetch PC becomes:
A) 199
B) 200
C) 201
D) 202

✅Answer: C) 201

🔸 7. Memory Reference Instructions

Q19. Which of the following is a memory reference instruction?
A) CLA
B) CMA
C) ADD
D) HLT

✅Answer: C) ADD

Q20. The microoperation for ADD instruction is:
A) AC ← AC + DR
B) DR ← AC
C) AC ← DR
D) PC ← PC + 1

✅Answer: A) AC ← AC + DR

🔸 8. Input-Output and Interrupt

Q21. Interrupt cycle is initiated when:
A) IEN = 0
B) FGI or FGO = 1 and IEN = 1
C) AC = 0
D) PC = 0

✅Answer: B) FGI or FGO = 1 and IEN = 1

Q22. The interrupt flip-flop is cleared during:
A) Fetch cycle
B) Execute cycle
C) Interrupt cycle
D) Decode

✅Answer: C) Interrupt cycle

Q23.DMA transfers data:
A) Through CPU only
B) Without CPU intervention
C) Using AC
D) Using IR

✅Answer: B) Without CPU intervention

🔸 9. LMC (Little Man Computer)

Q24. In LMC, memory consists of:
A) 10 locations
B) 50 locations
C) 100 locations
D) 256 locations

✅Answer: C) 100 locations

Q25. LMC instruction “901” represents:
A) INP
B) OUT
C) HLT
D) ADD

✅Answer: A) INP

Q26. In LMC, the accumulator is used for:
A) Storing instruction
B) Performing arithmetic operations
C) Storing address
D) Program counter

✅Answer: B) Performing arithmetic operations

If opcode field contains 5 bits, but only 28 instructions are defined, the number of unused opcodes is:
A) 2
B) 4
C) 6
D) 8

✅Answer: B) 4
(2⁵ = 32 → 32 − 28 = 4 unused)

During fetch cycle, which sequence of micro-operations is correct?
A) IR ← M[PC], PC ← PC + 1
B) AR ← PC, IR ← M[AR], PC ← PC + 1
C) DR ← PC, IR ← DR
D) PC ← IR

✅Answer: B

In basic computer, instruction fetch occurs during:
A) T0–T1
B) T0–T2
C) T1–T3
D) T2–T4

✅Answer: B

Q11. At T0, the microoperation performed is:
A) IR ← M[AR]
B) AR ← PC
C) PC ← PC + 1
D) DR ← AC

✅Answer: B

The effective address in indirect addressing is computed during:
A) Fetch
B) Decode
C) Execute
D) Interrupt

✅Answer: B

Q16. Which instruction affects E (carry) flip-flop?
A) AND
B) ADD
C) LDA
D) STA

✅Answer: B

A system uses 1-address instruction format. To compute X = (A + B) × C, minimum number of instructions required is:
A) 2
B) 3
C) 4
D) 5

✅Answer: C

A two-address instruction ADD R1, R2 performs:
A) R1 ← R1 + R2
B) R2 ← R1 + R2
C) R1 ← R2
D) R2 ← R1

✅Answer: A

Q15. The main disadvantage of three-address instruction is:
A) More memory required
B) More instructions required
C) Slower execution
D) No ALU

✅Answer: A

Interrupts improve system performance by:
A) Stopping CPU
B) Allowing CPU to perform other tasks
C) Reducing memory
D) Increasing clock speed

✅ Answer: B

Q20. When interrupt occurs, the address of next instruction is stored in:
A) IR
B) PC
C) Stack
D) DR

✅ Answer: C

Q21. Conditional branch instructions depend on:
A) ALU
B) Status flags
C) Memory
D) DMA

✅ Answer: B

🔹 8. Processor Status Word (PSW)

Q22. Processor status word contains:
A) Data only
B) Address only
C) Condition flags
D) Program

✅ Answer: C

Q23. Zero flag is set when:
A) Result is negative
B) Result is zero
C) Carry occurs
D) Overflow occurs

✅ Answer: B

Q24. Carry flag is affected during:
A) Logical operation only
B) Arithmetic operation
C) Data transfer
D) Branch instruction

✅ Answer: B

Q25. Overflow flag indicates:
A) Unsigned overflow
B) Signed arithmetic overflow
C) Zero result
D) Interrupt

✅ Answer: B

Evaluate postfix expression using stack:
5 3 2 * +
A) 11
B) 16
C) 21
D) 25

✅ Answer: A
(3×2 = 6 → 5+6 = 11)

If PC = 1000 and address field = 20 in relative addressing, effective address is:
A) 20
B) 1000
C) 1020
D) 980

✅ Answer: C

Q8. In base register addressing, effective address =
A) Base register + displacement
B) PC + displacement
C) Index + PC
D) Memory[Base]

✅ Answer: A

Q9. Which addressing mode requires two memory accesses to fetch operand?
A) Immediate
B) Direct
C) Indirect
D) Register

✅ Answer: C

🔹 4. RISC vs CISC (Conceptual Depth)

Q10. Which feature increases pipeline efficiency in RISC architecture?
A) Variable instruction length
B) Complex microprogramming
C) Fixed instruction format
D) Multiple addressing modes

✅ Answer: C

Q11. CISC architecture typically reduces:
A) Code size
B) Clock speed
C) Hardware complexity
D) Memory size

✅ Answer: A

Q12. Load-store architecture is a characteristic of:
A) CISC
B) RISC
C) Stack architecture
D) Microprogrammed CPU

✅ Answer: B

If PC = 1000 and address field = 20 in relative addressing, effective address is:
A) 20
B) 1000
C) 1020
D) 980

✅ Answer: C

Q8. In base register addressing, effective address =
A) Base register + displacement
B) PC + displacement
C) Index + PC
D) Memory[Base]

✅ Answer: A

Q9. Which addressing mode requires two memory accesses to fetch operand?
A) Immediate
B) Direct
C) Indirect
D) Register

✅ Answer: C

🔹 4. RISC vs CISC (Conceptual Depth)

Q10. Which feature increases pipeline efficiency in RISC architecture?
A) Variable instruction length
B) Complex microprogramming
C) Fixed instruction format
D) Multiple addressing modes

✅ Answer: C

Q11. CISC architecture typically reduces:
A) Code size
B) Clock speed
C) Hardware complexity
D) Memory size

✅ Answer: A

Q12. Load-store architecture is a characteristic of:
A) CISC
B) RISC
C) Stack architecture
D) Microprogrammed CPU

✅ Answer: B

A 32-bit instruction uses 8 bits for opcode and 3 register fields (5 bits each). Remaining bits available for immediate field are:

A) 9

B) 10

C) 11

D) 12

✅ Answer: A

(8 + 15 = 23 → 32 − 23 = 9)

If subtraction results in negative value, which flag is set?
A) Carry
B) Overflow
C) Sign
D) Zero

✅ Answer: C

If control word contains 5 bits for ALU operation and 3 bits each for selecting two source registers and one destination register (8 registers total), total control word size is:
A) 11 bits
B) 14 bits
C) 16 bits
D) 17 bits

✅ Answer: B
ALU = 5
Source1 = 3
Source2 = 3
Destination = 3
Total = 14 bits

If PC = 5000, index register = 200, base register = 1000, and address field = 50, effective address in base-index addressing is:
A) 1050
B) 1250
C) 5250
D) 5000

✅ Answer: B
EA = Base + Index + Displacement
= 1000 + 200 + 50

General Register Organization

Q1. In a system with 32 registers, how many bits are required to encode two source registers and one destination register?
A) 10
B) 12
C) 15
D) 18

✅ Answer: C) 15
(5 bits per register × 3)

Q2. In a bus-organized register system, if 8 registers share a common bus, the number of multiplexers required (assuming 16-bit registers) is:
A) 1
B) 8
C) 16
D) 32

✅ Answer: C) 16
(One MUX per bus line)

Q3. The main advantage of general register organization over accumulator organization is:
A) Fewer instructions
B) Reduced memory access
C) Smaller opcode
D) No ALU required

✅ Answer: B)

🔷 Stack Organization

Q4. In a memory stack, overflow condition occurs when:
A) SP = 0
B) SP exceeds stack limit
C) SP = PC
D) SP = IR

✅ Answer: B)

Q5. In a stack computer, expression evaluation of (A + B) × C requires how many PUSH operations?
A) 2
B) 3
C) 4
D) 5

✅ Answer: B)
(PUSH A, PUSH B, ADD, PUSH C, MUL → 3 PUSH)

Q6. A stack organized computer does not require:
A) ALU
B) Stack pointer
C) Address field in arithmetic instruction
D) Memory

✅ Answer: C)

🔷 Addressing Modes

Q7. If PC = 400, address field = 20, relative addressing gives effective address:
A) 20
B) 400
C) 420
D) 380

✅ Answer: C)

Q8. Auto-decrement addressing mode is typically used in:
A) Array traversal
B) Stack implementation
C) I/O transfer
D) Interrupt

✅ Answer: B)

Q9. Indexed addressing is useful for:
A) Subroutine calls
B) Array processing
C) Interrupt handling
D) Logical shift

✅ Answer: B)

🔷 Instruction Formats

Q10. Which instruction format minimizes memory traffic?
A) Three-address
B) Two-address
C) One-address
D) Register-to-register

✅ Answer: D)

Q11. In a two-address instruction:
ADD R1, R2
After execution:
A) R1 unchanged
B) R2 unchanged
C) R1 = R1 + R2
D) R2 = R1 + R2

✅ Answer: C)

Q12. Zero-address instruction format is primarily associated with:
A) Accumulator machine
B) Stack machine
C) RISC
D) DMA

✅ Answer: B)

🔷 RISC vs CISC

Q13. Which feature improves pipelining efficiency in RISC?
A) Variable instruction length
B) Few registers
C) Fixed instruction format
D) Complex addressing

✅ Answer: C)

Q14. CISC instructions often require:
A) Single cycle execution
B) Microprogrammed control memory
C) No memory access
D) No decoding

✅ Answer: B)

Q15. RISC architecture typically uses:
A) Memory-to-memory instructions
B) Load-store instructions
C) Stack-based execution
D) Microcoded branching

✅ Answer: B)

🔷 Data Transfer Schemes

Q16. In programmed I/O, the CPU wastes time because:
A) It executes DMA
B) It continuously polls device
C) It resets PC
D) It clears memory

✅ Answer: B)

Q17. DMA controller generates interrupt after:
A) Every byte transfer
B) Completion of block transfer
C) Instruction fetch
D) ALU operation

✅ Answer: B)

Q18. In interrupt-driven I/O, the interrupt service routine address is stored in:
A) Data register
B) Interrupt vector
C) Stack pointer
D) IR

✅ Answer: B)

🔷 Program Control & Interrupts

Q19. The instruction cycle is modified by interrupt by inserting:
A) Fetch cycle
B) Execute cycle
C) Interrupt cycle
D) Memory cycle

✅ Answer: C)

Q20. Nested interrupts require:
A) Single priority
B) Interrupt masking mechanism
C) No stack
D) No PSW

✅ Answer: B)

Q21. The main function of interrupt enable flip-flop is to:
A) Enable ALU
B) Enable memory
C) Control acceptance of interrupts
D) Store return address

✅ Answer: C)

🔷 Processor Status Word (PSW)

Q22. Sign flag reflects:
A) Carry out
B) MSB of result
C) LSB of result
D) Interrupt

✅ Answer: B)

Q23. Which flags are affected by arithmetic operations?
A) Zero, Carry, Sign, Overflow
B) PC only
C) IR only
D) MAR only

✅ Answer: A)

Q24. If result of signed addition exceeds range, which flag is set?
A) Carry
B) Zero
C) Overflow
D) Interrupt

✅ Answer: C)

Q25. Processor status word is stored:
A) In ALU
B) In control unit register
C) In memory only
D) In cache

✅ Answer: B)

Q1.

In Mano’s Basic Computer, during fetch cycle, the microoperations are:

T0: AR ← PC
T1: IR ← M[AR], PC ← PC + 1
T2: Decode IR

At which timing signal is the instruction placed in IR?

A) T0
B) T1
C) T2
D) T3

✅ Answer: B

Q2.

If instruction is memory-reference and I = 1 (indirect), effective address is computed at:

A) T1
B) T2
C) T3
D) T4

✅ Answer: C

Q3.

During execution of AND instruction, AC ← AC ∧ M[AR] occurs at:

A) T3
B) T4
C) T5
D) T6

✅ Answer: B (T4)

🔥 PART 2: Effective Address (Numerical)

Q4.

Given:
PC = 500
Address field = 20
Relative addressing mode

Effective Address = ?

A) 480
B) 500
C) 520
D) 5200

✅ Answer: C

Q5.

Given:
Address field = 300
M[300] = 900
Indirect addressing

Effective Address = ?

A) 300
B) 900
C) M[900]
D) PC

✅ Answer: B

Q6.

If R1 = 1000 and address field = 50 in indexed addressing:

EA = ?

A) 950
B) 1000
C) 1050
D) 50

✅ Answer: C

Q7.

Stack grows downward.
SP = 500
After PUSH, PUSH, POP

Final SP = ?

A) 498
B) 499
C) 500
D) 501

✅ Answer: B
(500 → 499 → 498 → 499)

Q8.

In stack machine, postfix expression:
A B + C ×

How many arithmetic operations are executed?

A) 1
B) 2
C) 3
D) 4

✅ Answer: B

Q9.

Stack overflow occurs when:

A) SP = 0
B) Stack full and PUSH executed
C) POP executed
D) PC = SP

✅ Answer: B

Q10.

Three-address instruction requires how many bits for register selection if 32 registers exist?

A) 10
B) 12
C) 15
D) 20

✅ Answer: C (5×3)

Q11.

Which instruction format causes maximum memory traffic?

A) Register-register
B) Zero-address
C) Three-address memory
D) Load-store

✅ Answer: C

Q12.

Which feature of RISC reduces control complexity?

A) Microprogramming
B) Fixed instruction length
C) Complex addressing modes
D) Memory-to-memory instructions

✅ Answer: B

Q13.

CISC architecture often suffers from pipeline hazards due to:

A) Few registers
B) Variable instruction length
C) Fixed format
D) Load-store design

✅ Answer: B

Q14.

During interrupt cycle, the return address is:

A) Stored in IR
B) Stored in memory location 0
C) Stored in stack
D) Cleared

✅ Answer: C

Q15.

Interrupt enable flip-flop = 0 means:

A) Interrupts accepted
B) Interrupts masked
C) DMA active
D) ALU disabled

✅ Answer: B

Q16.

Priority interrupt is required when:

A) Single I/O device
B) Multiple devices request simultaneously
C) Stack empty
D) No control unit

✅ Answer: B

Q17.

If result = 00000000, flags affected:

A) Zero = 1
B) Carry = 1
C) Overflow = 1
D) Sign = 1

✅ Answer: A

Q18.

Signed addition overflow occurs when:

A) Adding two positive numbers gives negative result
B) Carry generated
C) Zero result
D) Interrupt

✅ Answer: A

Q19.

Sign flag is determined by:

A) Carry bit
B) MSB of result
C) LSB of result
D) Stack pointer

✅ Answer: B

Q20.

In Basic Computer, if instruction is:

ADD 500 (Indirect)

Sequence includes:

A) Fetch → Decode → Execute
B) Fetch → Decode → Indirect → Execute
C) Decode → Execute
D) Execute only

✅ Answer: B

Q21.

In Basic Computer, how many clock cycles are required to execute memory-reference instruction with indirect addressing?

A) 3
B) 4
C) 5
D) 6

✅ Answer: D
(T0–T5)

Q22.

Which architecture eliminates memory operands in arithmetic instructions?

A) CISC
B) Stack
C) RISC
D) Accumulator

✅ Answer: C

Q23.

In general register organization, number of control bits increases with:

A) Memory size
B) Number of registers
C) Clock speed
D) Stack depth

✅ Answer: B

If IR(15)=1 and opcode = 001, the instruction belongs to:

A) Register reference
B) I/O instruction
C) Indirect memory reference
D) Direct memory reference

✅ Answer: C

If PC = 450, instruction address field = 200, and relative addressing is used, the effective address is:

A) 200
B) 450
C) 650
D) 250

✅ Answer: C

If SP = 500 and PUSH is executed (stack grows downward), the new SP becomes:

A) 501
B) 499
C) 500
D) 498

✅ Answer: B

Interrupt cycle begins when:

A) IEN = 0
B) R = 1 and SC = 0
C) FGI = 0
D) PC = 0

✅ Answer: B

If a computer has 64 registers and uses 3-address instructions, minimum bits required only for register specification:

A) 12
B) 15
C) 18
D) 6

✅ Answer: C
(6 bits per register × 3 = 18 bits)

Why is RISC more pipeline-friendly than CISC?

A) Uses microprogramming
B) Variable length instructions
C) Fixed instruction format
D) Complex addressing modes

✅ Answer: C

If AC = 1111 1111 1111 1111 and ADD 0000 0000 0000 0001 is executed (16-bit signed), which flags will be set?

A) Zero only
B) Carry only
C) Carry and Zero
D) Carry and Overflow

✅ Answer: C

If memory size increases, instruction length must increase to accommodate:

A) Opcode
B) Register bits
C) Address field
D) PSW

✅ Answer: C

Three-bus organization allows:

A) One register read
B) Two register reads + one write
C) Only ALU operations
D) Only memory access

✅ Answer: B

If a computer has 4K memory and 16-bit instruction format with 1 indirect bit and 3-bit opcode, remaining bits for address:

A) 10
B) 11
C) 12
D) 13

✅ Answer: C
(16 - 1 - 3 = 12 bits → 4K memory)

If during execution of a memory-reference instruction, the sequence counter (SC) is not cleared at T6, what will happen?

A) CPU halts
B) Next instruction fetch is skipped
C) Timing sequence continues abnormally
D) Interrupt cycle begins

✅ Answer: C

In Basic Computer, the instruction BSA X performs:

A) AC ← M[X]
B) M[X] ← PC, PC ← X + 1
C) PC ← X
D) AC ← AC + M[X]

✅ Answer: B

If PC = 325 and instruction is BUN 500, after execution PC becomes:

A) 326
B) 500
C) 501
D) 825

✅ Answer: B

Auto-increment addressing mode is mainly used in:

A) Arithmetic
B) Loop operations
C) Stack operations
D) DMA

✅ Answer: B

Instruction: MOV R1, #25
This is:

A) Direct
B) Register indirect
C) Immediate
D) Indexed

✅ Answer: C

RISC architecture typically reduces CPI because:

A) Instructions are simple
B) Uses microprogramming
C) Variable length
D) More addressing modes

✅Answer: A

A system has 8K memory. Minimum address bits required:

A) 12
B) 13
C) 14
D) 16

✅Answer: B
(2¹³ = 8192 ≈ 8K)

Unit 4

Which step in the HDL design flow converts high-level RTL code into a gate-level netlist?
A. Simulation
B. Synthesis
C. Place and Route
D. Verification
Answer: B. Synthesis

Which of the following is a Micro-operation?

Arithmetic
B. Logic
C. Shift
D. All of the above
Answer: D. All of the above

Which statement accurately describes a register transfer language (RTL)?

A high-level language like C++
B. A language to describe software code
C. A symbolic language used to describe the micro-operation sequences among registers
D. A language to describe PCB layout

What is the purpose of the Arithmetic Shift Right (ASR) micro-operation?
A. To rotate bits
B. To multiply by 2
C. To divide by 2 while preserving the sign bit
D. To clear the register
Answer: C. To divide by 2 while preserving the sign bit

Which adder circuit is faster?
A. Ripple Carry Adder
B. Carry Lookahead Adder
C. Serial Adder
D. Subtractor
Answer: B. Carry Lookahead Adder

13. Which of the following is true about HDL compared to software programming languages?
A. HDL code executes sequentially like C
B. HDL represents parallel hardware structure, while software is sequential
C. HDL does not support simulation
D. HDL is only used for analog simulation
Answer: B. HDL represents parallel hardware structure, while software is sequential

Q1. HDL stands for:
A) High Design Logic
B) Hardware Description Language
C) High Data Language
D) Hardware Development Logic
Answer: B

Q2. Which HDL is widely used for digital system design?
A) C++
B) Python
C) Verilog
D) Java
Answer: C

Q3. The first step in HDL design flow is:
A) Synthesis
B) Simulation
C) Design entry
D) Verification
Answer: C

Q4. Simulation is mainly used to:
A) Convert code to hardware
B) Check functionality
C) Reduce hardware cost
D) Optimize speed
Answer: B

Q7. HDL differs from software languages because it:
A) Executes sequentially only
B) Describes hardware behavior
C) Cannot use loops
D) Is slower
Answer: B

Q8. HDL supports:
A) Only sequential execution
B) Parallel execution
C) No execution
D) Only arithmetic
Answer: B

Q11. Register Transfer means:
A) Data transfer between memory
B) Data transfer between registers
C) Data transfer to CPU only
D) Data transfer to I/O
Answer: B

Q12. Micro-operations operate on:
A) Programs
B) Registers
C) Files
D) Instructions
Answer: B

Q19. Booth’s algorithm is used for:
A) Division
B) Fast multiplication of signed numbers
C) Sorting
D) Searching
Answer: B

Q20. In binary multiplication, shifting left is equivalent to:
A) Division by 2
B) Multiplication by 2
C) Addition
D) Subtraction
Answer: B

Unit 5

- Which memory type is fastest and located closest to the CPU?
a) Main Memory (RAM)
b) Cache Memory
c) Registers
d) Secondary Memory
Answer: c) Registers

- 2. Which of the following is considered "auxiliary memory"?
a) RAM
b) ROM
c) Cache
d) Hard Disk Drive
Answer: d) Hard Disk Drive

- 4. What is the primary purpose of Virtual Memory?
a) To make RAM faster
b) To increase the overall memory capacity beyond physical RAM
c) To provide permanent storage
d) To speed up the CPU
Answer: b) To increase the overall memory capacity beyond physical

- 7. In a 4-way set-associative cache, how many lines are in each set?
a) 1
b) 2
c) 4
d) 8
Answer: c) 4

- 8. What is the "dirty bit" used for in cache management?
a) To indicate a cache miss
b) To indicate that the block has been modified
c) To lock the block
d) To clear the cache
Answer: b) To indicate that the block has been modified

- Q1. The correct order of memory hierarchy (fastest to slowest) is:
A) Cache → Register → Main Memory → Secondary Memory
B) Register → Cache → Main Memory → Secondary Memory
C) Main Memory → Cache → Register → Secondary Memory
D) Register → Main Memory → Cache → Secondary Memory
Answer: B

- Q2. The purpose of memory hierarchy is to:
A) Increase CPU speed only
B) Reduce cost and access time
C) Increase storage only
D) Eliminate RAM
Answer: B

- Q3. Cache memory is:
A) Slower than RAM
B) Faster than RAM
C) Same as ROM
D) External memory
Answer: B

- Q4. Cache memory is located:
A) Inside CPU or close to CPU
B) In hard disk
C) In RAM
D) In ROM
Answer: A

- Q5. Cache improves performance by:
A) Increasing memory size
B) Reducing access time
C) Reducing instructions
D) Increasing clock speed
Answer: B

- Q11. Virtual memory provides:
A) Larger logical memory than physical memory
B) Smaller memory
C) Only RAM
D) Only disk storage
Answer: A

- Q12. Virtual memory uses:
A) Cache only
B) Hard disk as extension of RAM
C) Registers
D) ROM
Answer: B

- Q13. Paging is used in:
A) Cache memory
B) Virtual memory
C) Registers
D) ALU
Answer: B

- Q16. Parallel processing means:
A) Single task at a time
B) Multiple tasks simultaneously
C) Only sequential execution
D) No execution
Answer: B

- Q17. Main goal of parallel processing is:
A) Reduce cost
B) Increase speed and performance
C) Reduce memory
D) Increase power consumption
Answer: B

- Unit 6

- Q1. NVIDIA is primarily known for:
A) Operating systems
B) Graphics Processing Units (GPUs)
C) Hard disks
D) Compilers
Answer: B

- Q2. GPUs are mainly used for:
A) Sequential processing
B) Parallel processing
C) Data storage
D) Networking
Answer: B

- Q3. CUDA is developed by NVIDIA for:
A) Networking
B) Parallel computing
C) Storage
D) Security
Answer: B

## 2. Introduction to Supercomputers

- Q4. A supercomputer is designed for:
A) Gaming
B) High-performance computing
C) Office work
D) Web browsing
Answer: B

- Q5. Which is a famous supercomputer?
A) Fugaku
B) Dell Laptop
C) iPhone
D) Printer
Answer: A

- Q6. Supercomputers use:
A) Single processor
B) Multiple processors
C) No processors
D) Only GPU
Answer: B

## 3. Qubits & Quantum Computing

- Q7. A qubit can represent:
A) Only 0
B) Only 1
C) Both 0 and 1 simultaneously
D) Neither
Answer: C

- Q8. Quantum computing uses principles of:
A) Classical physics
B) Quantum mechanics
C) Thermodynamics only
D) Mechanics only
Answer: B

- Q9. Superposition means:
A) Single state
B) Multiple states at once
C) No state
D) Fixed state
Answer: B

- Q10. Entanglement refers to:
A) Independent particles
B) Linked quantum states
C) Data storage
D) Memory hierarchy
Answer: B

## 4. Latest Trends in Computer Architecture

- Q11. AI accelerators are used for:
A) Storage
B) Machine learning tasks
C) Networking
D) Cooling
Answer: B

- Q12. Edge computing means:
A) Computing in cloud only
B) Processing near data source
C) Only CPU processing
D) Offline computing
Answer: B

- Q13. Heterogeneous computing involves:
A) Same processors
B) Different types of processors
C) No processors
D) Only GPU
Answer: B

## 5. Next Generation Processor Architecture

- Q14. Multi-core processors contain:
A) One core
B) Multiple cores
C) No core
D) Only GPU
Answer: B

- Q15. ARM architecture is commonly used in:
A) Supercomputers
B) Smartphones
C) Mainframes
D) Servers only
Answer: B

- Q16. RISC architecture emphasizes:
A) Complex instructions
B) Simple instructions
C) No instructions
D) Only memory
Answer: B

## 6. Microarchitecture

- Q17. Microarchitecture refers to:
A) Software design
B) Internal CPU design
C) Network design
D) Database design
Answer: B

- Q18. Pipeline stages include:
A) Fetch, Decode, Execute
B) Store only
C) Compile only
D) None
Answer: A

## 7. Latest Processors (Smartphone/Tablet)

- Q19. Apple A17 Pro is used in:
A) Desktop PCs
B) Smartphones
C) Servers
D) Routers
Answer: B

- Q20. Snapdragon 8 Gen 3 is developed by:
A) Intel
B) AMD
C) Qualcomm
D) IBM
Answer: C

- Q21. Mobile processors focus on:
A) High power consumption
B) Power efficiency
C) Large size
D) Cooling only
Answer: B

## 8. Latest Desktop Processors

- Q22. Intel processors are known for:
A) GPUs only
B) CPUs
C) Storage devices
D) Networking
Answer: B

- Q23. AMD Ryzen processors are:
A) GPUs
B) CPUs
C) RAM
D) SSD
Answer: B

- Q24. Desktop processors prioritize:
A) Battery life
B) Performance
C) Small size
D) Mobility
Answer: B

## 9. Advanced Concepts

- Q25. GPU parallelism is useful in:
A) Word processing
B) Scientific computing
C) Printing
D) Typing
Answer: B

- Q26. Quantum computers are best suited for:
A) Simple arithmetic
B) Complex optimization problems
C) Typing documents
D) Browsing
Answer: B

## 10. General Understanding

- Q27. Moore’s Law refers to:
A) Memory size
B) Transistor doubling
C) CPU cooling
D) Networking speed
Answer: B

- Q28. Chiplet architecture helps in:
A) Reducing flexibility
B) Modular chip design
C) Increasing size only
D) Reducing cores
Answer: B

- Q29. Tensor cores are used for:
A) Graphics only
B) AI computations
C) Storage
D) Networking
Answer: B

- Q30. Future processors aim for:
A) Lower efficiency
B) Higher performance and efficiency
C) Larger size only
D) No improvement
Answer: B

1. Which of the following is an input device?
A. Printer
B. Monitor
C. Keyboard
D. Speaker
✅ Answer: C

2. Which device is both input and output?
A. Scanner
B. Touchscreen
C. Plotter
D. Speaker
✅ Answer: B

3. Hard disk is an example of:
A. Primary memory
B. Secondary storage
C. Input device
D. Output device
✅ Answer: B

4. Which device provides hard copy output?
A. Monitor
B. Printer
C. Keyboard
D. Mouse
✅ Answer: B

5. Which is NOT a peripheral device?
A. CPU
B. Keyboard
C. Printer
D. Scanner
✅ Answer: A

6. A scanner converts:
A. Hard copy to soft copy
B. Soft copy to hard copy
C. Analog to digital
D. Digital to analog
✅ Answer: A

7. Which peripheral stores data permanently?
A. RAM
B. Cache
C. Hard Disk
D. Register
✅ Answer: C

## ⚡ Priority Interrupt (8–14)

8. Interrupt is a signal that:
A. Stops CPU permanently
B. Requests CPU attention
C. Deletes memory
D. None
✅ Answer: B

9. Priority interrupt is used to:
A. Ignore interrupts
B. Assign importance levels
C. Reduce memory
D. Increase speed
✅ Answer: B

10. Highest priority interrupt is serviced:
A. Last
B. First
C. Randomly
D. Never
✅ Answer: B

11. Daisy chaining is used in:
A. Memory
B. Interrupt handling
C. ALU
D. Registers
✅ Answer: B

12. Which hardware is used for priority interrupt?
A. Encoder
B. Decoder
C. Multiplexer
D. Flip-flop
✅ Answer: A

13. Maskable interrupts can be:
A. Ignored
B. Deleted
C. Stored
D. Executed
✅ Answer: A

14. Non-maskable interrupts are:
A. Ignorable
B. High priority
C. Low priority
D. Temporary
✅ Answer: B

## 🔌 Input/Output Interface (15–21)

15. I/O interface connects:
A. CPU and memory
B. CPU and peripheral
C. Memory and disk
D. ALU and CU
✅ Answer: B

16. Interface is required because of:
A. Speed mismatch
B. Size mismatch
C. Voltage mismatch
D. All
✅ Answer: D

17. Which register holds data during transfer?
A. Address register
B. Data register
C. Control register
D. Status register
✅ Answer: B

18. Status register indicates:
A. Data value
B. Device state
C. Address
D. Instruction
✅ Answer: B

19. Control register is used to:
A. Store data
B. Send commands
C. Store address
D. None
✅ Answer: B

20. I/O mapped I/O uses:
A. Separate address space
B. Same address space
C. Cache memory
D. Virtual memory
✅ Answer: A

21. Memory mapped I/O uses:
A. Separate space
B. Common address space
C. No address
D. External memory
✅ Answer: B

## 🚀 Direct Memory Access (DMA) (22–28)

22. DMA allows data transfer between:
A. CPU and memory
B. I/O and memory
C. CPU and I/O
D. ALU and memory
✅ Answer: B

23. DMA reduces:
A. CPU involvement
B. Memory size
C. Speed
D. Interrupts
✅ Answer: A

24. DMA controller controls:
A. ALU
B. Registers
C. Data transfer
D. Cache
✅ Answer: C

25. Cycle stealing is related to:
A. CPU
B. DMA
C. Cache
D. ALU
✅ Answer: B

26. Burst mode in DMA transfers:
A. One byte
B. Block of data
C. No data
D. Random data
✅ Answer: B

27. DMA request is sent by:
A. CPU
B. Memory
C. I/O device
D. ALU
✅ Answer: C

28. DMA acknowledge is sent by:
A. CPU
B. Memory
C. Device
D. ALU
✅ Answer: A

## 🧠 Input/Output Processor (29–35)

29. I/O processor is used to:
A. Replace CPU
B. Handle I/O tasks
C. Increase RAM
D. Reduce memory
✅ Answer: B

30. IOP operates:
A. Independently
B. Only with CPU
C. Only with memory
D. None
✅ Answer: A

31. IOP improves:
A. CPU load
B. System performance
C. Memory size
D. Power
✅ Answer: B

32. IOP executes:
A. Arithmetic ops
B. I/O instructions
C. Logical ops
D. None
✅ Answer: B

33. IOP communicates with CPU via:
A. Interrupt
B. Cache
C. ALU
D. Register
✅ Answer: A

34. IOP has its own:
A. Memory
B. Program
C. Control unit
D. All
✅ Answer: D

35. IOP reduces:
A. CPU workload
B. Memory size
C. Interrupts
D. Speed
✅ Answer: A

## 🔄 Modes of Data Transfer (36–42)

36. Programmed I/O requires:
A. CPU involvement
B. DMA
C. Interrupt
D. None
✅ Answer: A

37. Interrupt-driven I/O reduces:
A. CPU idle time
B. Memory
C. Speed
D. Registers
✅ Answer: A

38. DMA mode transfers data:
A. Through CPU
B. Directly
C. Slowly
D. Indirectly
✅ Answer: B

39. Fastest mode of transfer is:
A. Programmed
B. Interrupt
C. DMA
D. Manual
✅ Answer: C

40. In programmed I/O CPU waits:
A. Actively
B. Passively
C. Never
D. Randomly
✅ Answer: A

41. Interrupt-driven I/O uses:
A. Polling
B. Interrupt signal
C. DMA
D. Cache
✅ Answer: B

42. Polling is used in:
A. DMA
B. Programmed I/O
C. Interrupt
D. None
✅ Answer: B

## 📡 UART (43–50)

43. UART stands for:
A. Universal Asynchronous Receiver Transmitter
B. Unified Automatic Receiver Transfer
C. Universal Analog Receiver Transmitter
D. None
✅ Answer: A

44. UART is used for:
A. Parallel communication
B. Serial communication
C. Memory
D. Processing
✅ Answer: B

45. UART converts:
A. Parallel to serial
B. Serial to parallel
C. Both
D. None
✅ Answer: C

46. UART uses:
A. Clock signal
B. No clock (asynchronous)
C. Cache
D. ALU
✅ Answer: B

47. Start bit in UART is:
A. 0
B. 1
C. Both
D. None
✅ Answer: A

48. Stop bit in UART is:
A. 0
B. 1
C. Both
D. None
✅ Answer: B

49. Baud rate defines:
A. Speed of data transfer
B. Memory size
C. Voltage
D. Current
✅ Answer: A

50. Parity bit is used for:
A. Speed
B. Error detection
C. Memory
D. Control
✅ Answer: B

1. Which of the following is an input device?
A. Printer
B. Monitor
C. Keyboard
D. Speaker
✅ Answer: C

2. Which device is both input and output?
A. Scanner
B. Touchscreen
C. Plotter
D. Speaker
✅ Answer: B

3. Hard disk is an example of:
A. Primary memory
B. Secondary storage
C. Input device
D. Output device
✅ Answer: B

4. Which device provides hard copy output?
A. Monitor
B. Printer
C. Keyboard
D. Mouse
✅ Answer: B

5. Which is NOT a peripheral device?
A. CPU
B. Keyboard
C. Printer
D. Scanner
✅ Answer: A

6. A scanner converts:
A. Hard copy to soft copy
B. Soft copy to hard copy
C. Analog to digital
D. Digital to analog
✅ Answer: A

7. Which peripheral stores data permanently?
A. RAM
B. Cache
C. Hard Disk
D. Register
✅ Answer: C

## ⚡ Priority Interrupt (8–14)

8. Interrupt is a signal that:
A. Stops CPU permanently
B. Requests CPU attention
C. Deletes memory
D. None
✅ Answer: B

9. Priority interrupt is used to:
A. Ignore interrupts
B. Assign importance levels
C. Reduce memory
D. Increase speed
✅ Answer: B

10. Highest priority interrupt is serviced:
A. Last
B. First
C. Randomly
D. Never
✅ Answer: B

11. Daisy chaining is used in:
A. Memory
B. Interrupt handling
C. ALU
D. Registers
✅ Answer: B

12. Which hardware is used for priority interrupt?
A. Encoder
B. Decoder
C. Multiplexer
D. Flip-flop
✅ Answer: A

13. Maskable interrupts can be:
A. Ignored
B. Deleted
C. Stored
D. Executed
✅ Answer: A

14. Non-maskable interrupts are:
A. Ignorable
B. High priority
C. Low priority
D. Temporary
✅ Answer: B

## 🔌 Input/Output Interface (15–21)

15. I/O interface connects:
A. CPU and memory
B. CPU and peripheral
C. Memory and disk
D. ALU and CU
✅ Answer: B

16. Interface is required because of:
A. Speed mismatch
B. Size mismatch
C. Voltage mismatch
D. All
✅ Answer: D

17. Which register holds data during transfer?
A. Address register
B. Data register
C. Control register
D. Status register
✅ Answer: B

18. Status register indicates:
A. Data value
B. Device state
C. Address
D. Instruction
✅ Answer: B

19. Control register is used to:
A. Store data
B. Send commands
C. Store address
D. None
✅ Answer: B

20. I/O mapped I/O uses:
A. Separate address space
B. Same address space
C. Cache memory
D. Virtual memory
✅ Answer: A

21. Memory mapped I/O uses:
A. Separate space
B. Common address space
C. No address
D. External memory
✅ Answer: B

## 🚀 Direct Memory Access (DMA) (22–28)

22. DMA allows data transfer between:
A. CPU and memory
B. I/O and memory
C. CPU and I/O
D. ALU and memory
✅ Answer: B

23. DMA reduces:
A. CPU involvement
B. Memory size
C. Speed
D. Interrupts
✅ Answer: A

24. DMA controller controls:
A. ALU
B. Registers
C. Data transfer
D. Cache
✅ Answer: C

25. Cycle stealing is related to:
A. CPU
B. DMA
C. Cache
D. ALU
✅ Answer: B

26. Burst mode in DMA transfers:
A. One byte
B. Block of data
C. No data
D. Random data
✅ Answer: B

27. DMA request is sent by:
A. CPU
B. Memory
C. I/O device
D. ALU
✅ Answer: C

28. DMA acknowledge is sent by:
A. CPU
B. Memory
C. Device
D. ALU
✅ Answer: A

## 🧠 Input/Output Processor (29–35)

29. I/O processor is used to:
A. Replace CPU
B. Handle I/O tasks
C. Increase RAM
D. Reduce memory
✅ Answer: B

30. IOP operates:
A. Independently
B. Only with CPU
C. Only with memory
D. None
✅ Answer: A

31. IOP improves:
A. CPU load
B. System performance
C. Memory size
D. Power
✅ Answer: B

32. IOP executes:
A. Arithmetic ops
B. I/O instructions
C. Logical ops
D. None
✅ Answer: B

33. IOP communicates with CPU via:
A. Interrupt
B. Cache
C. ALU
D. Register
✅ Answer: A

34. IOP has its own:
A. Memory
B. Program
C. Control unit
D. All
✅ Answer: D

35. IOP reduces:
A. CPU workload
B. Memory size
C. Interrupts
D. Speed
✅ Answer: A

## 🔄 Modes of Data Transfer (36–42)

36. Programmed I/O requires:
A. CPU involvement
B. DMA
C. Interrupt
D. None
✅ Answer: A

37. Interrupt-driven I/O reduces:
A. CPU idle time
B. Memory
C. Speed
D. Registers
✅ Answer: A

38. DMA mode transfers data:
A. Through CPU
B. Directly
C. Slowly
D. Indirectly
✅ Answer: B

39. Fastest mode of transfer is:
A. Programmed
B. Interrupt
C. DMA
D. Manual
✅ Answer: C

40. In programmed I/O CPU waits:
A. Actively
B. Passively
C. Never
D. Randomly
✅ Answer: A

41. Interrupt-driven I/O uses:
A. Polling
B. Interrupt signal
C. DMA
D. Cache
✅ Answer: B

42. Polling is used in:
A. DMA
B. Programmed I/O
C. Interrupt
D. None
✅ Answer: B

## 📡 UART (43–50)

43. UART stands for:
A. Universal Asynchronous Receiver Transmitter
B. Unified Automatic Receiver Transfer
C. Universal Analog Receiver Transmitter
D. None
✅ Answer: A

44. UART is used for:
A. Parallel communication
B. Serial communication
C. Memory
D. Processing
✅ Answer: B

45. UART converts:
A. Parallel to serial
B. Serial to parallel
C. Both
D. None
✅ Answer: C

46. UART uses:
A. Clock signal
B. No clock (asynchronous)
C. Cache
D. ALU
✅ Answer: B

47. Start bit in UART is:
A. 0
B. 1
C. Both
D. None
✅ Answer: A

48. Stop bit in UART is:
A. 0
B. 1
C. Both
D. None
✅ Answer: B

49. Baud rate defines:
A. Speed of data transfer
B. Memory size
C. Voltage
D. Current
✅ Answer: A

50. Parity bit is used for:
A. Speed
B. Error detection
C. Memory
D. Control
✅ Answer: B

If you want, I can also:

- Convert these into a PDF/notes format

- Add difficulty levels (easy/medium/hard)

- Or create previous-year exam style questions 👍

History, Evolution, Typical HDL-based Design Flow (Design, Simulation, Synthesis, Verification), Comparison with Software Programming Languages, Verilog, Register Transfer and Micro Operation, Register Transfer Language, Bus and Memory Transfer, Shift Register, Micro operations (Arithmetic, logic, shift), Addition, Subtraction, Multiplication Algorithm, Booth Multiplication

Here are 50 MCQs (with answers) covering your topics: History & Evolution, HDL Design Flow, Verilog, RTL, Micro-operations, Bus/Memory Transfer, Arithmetic Operations, and Booth Multiplication.

# 📘 Multiple Choice Questions

## 🕰️ History & Evolution of HDL (1–6)

1. HDL stands for:
A. High Data Language
B. Hardware Description Language
C. High Design Logic
D. Hardware Development Logic
✅ Answer: B

2. Which HDL was developed by the U.S. Department of Defense?
A. Verilog
B. VHDL
C. SystemC
D. Python
✅ Answer: B

3. Verilog was developed by:
A. IBM
B. Intel
C. Gateway Design Automation
D. Microsoft
✅ Answer: C

4. HDL is mainly used for:
A. Software development
B. Hardware modeling
C. Web design
D. Database management
✅ Answer: B

5. Evolution of HDL is driven by:
A. Faster CPUs
B. Complex digital circuits
C. Software bugs
D. Internet
✅ Answer: B

6. Which came first?
A. Verilog
B. VHDL
C. SystemVerilog
D. C++
✅ Answer: B

## ⚙️ HDL Design Flow (7–12)

7. First step in HDL design flow is:
A. Synthesis
B. Simulation
C. Design entry
D. Verification
✅ Answer: C

8. Simulation is used to:
A. Build hardware
B. Test design behavior
C. Reduce cost
D. Increase speed
✅ Answer: B

9. Synthesis converts HDL into:
A. Software
B. Gate-level netlist
C. Assembly code
D. Machine code
✅ Answer: B

10. Verification ensures:
A. Speed
B. Correctness
C. Memory
D. Cost
✅ Answer: B

11. Post-synthesis simulation checks:
A. Logic errors
B. Timing errors
C. Syntax errors
D. None
✅ Answer: B

12. Which tool is used before synthesis?
A. Compiler
B. Simulator
C. Assembler
D. Linker
✅ Answer: B

## 💻 HDL vs Software Languages (13–17)

13. HDL describes:
A. Instructions
B. Hardware behavior
C. Data structures
D. OS
✅ Answer: B

14. Software languages execute:
A. Sequentially
B. Concurrently
C. Randomly
D. Parallel only
✅ Answer: A

15. HDL supports:
A. Sequential only
B. Concurrent operations
C. No execution
D. Linear execution
✅ Answer: B

16. Example of software language:
A. Verilog
B. VHDL
C. C
D. RTL
✅ Answer: C

17. HDL is closer to:
A. Hardware
B. Software
C. OS
D. Database
✅ Answer: A

## 🔤 Verilog Basics (18–23)

18. Verilog is a:
A. Programming language
B. HDL
C. OS
D. Compiler
✅ Answer: B

19. Basic building block in Verilog:
A. Class
B. Module
C. Function
D. Loop
✅ Answer: B

20. Verilog supports:
A. Sequential only
B. Concurrent execution
C. No execution
D. Single thread
✅ Answer: B

21. ‘always’ block is used for:
A. Continuous assignment
B. Sequential logic
C. Memory
D. Input
✅ Answer: B

22. ‘assign’ is used for:
A. Sequential logic
B. Continuous assignment
C. Loop
D. Condition
✅ Answer: B

23. Verilog data type for storing value:
A. wire
B. reg
C. int
D. char
✅ Answer: B

## 🔄 Register Transfer & RTL (24–28)

24. RTL stands for:
A. Register Transfer Level
B. Real Time Logic
C. Register Timing Logic
D. None
✅ Answer: A

25. Register transfer means:
A. Data storage
B. Data movement between registers
C. Arithmetic operation
D. Logic design
✅ Answer: B

26. RTL describes:
A. Software
B. Hardware at register level
C. Memory
D. OS
✅ Answer: B

27. Control signals in RTL determine:
A. Data movement
B. Memory size
C. Speed
D. Voltage
✅ Answer: A

28. Micro-operations occur:
A. Between registers
B. In memory
C. In ALU only
D. In CPU only
✅ Answer: A

## 🔌 Bus & Memory Transfer (29–32)

29. Bus is used to:
A. Store data
B. Transfer data
C. Process data
D. Delete data
✅ Answer: B

30. System bus consists of:
A. Address, data, control
B. ALU, CU, memory
C. Input, output
D. Registers
✅ Answer: A

31. Memory transfer involves:
A. CPU and memory
B. ALU only
C. Registers only
D. Cache only
✅ Answer: A

32. Read operation means:
A. Write to memory
B. Fetch from memory
C. Delete memory
D. Copy memory
✅ Answer: B

## 🔁 Shift Register (33–36)

33. Shift register is used for:
A. Storage only
B. Data shifting
C. Arithmetic only
D. Logic only
✅ Answer: B

34. Types of shift registers:
A. SISO
B. SIPO
C. PISO
D. All
✅ Answer: D

35. Shift left operation multiplies by:
A. 2
B. 4
C. 8
D. 16
✅ Answer: A

36. Shift right divides by:
A. 2
B. 4
C. 8
D. 16
✅ Answer: A

## ➕ Micro Operations (Arithmetic, Logic, Shift) (37–42)

37. Arithmetic operation includes:
A. AND
B. OR
C. ADD
D. NOT
✅ Answer: C

38. Logical operation includes:
A. ADD
B. SUB
C. AND
D. SHIFT
✅ Answer: C

39. Shift operation includes:
A. Rotate
B. Add
C. Multiply
D. Divide
✅ Answer: A

40. Subtraction can be done using:
A. Complement
B. Addition
C. Both
D. None
✅ Answer: C

41. 2’s complement is used for:
A. Addition
B. Subtraction
C. Multiplication
D. Division
✅ Answer: B

42. ALU performs:
A. Arithmetic & logic
B. Storage
C. Input
D. Output
✅ Answer: A

## ✖️ Multiplication Algorithms (43–46)

43. Basic multiplication uses:
A. Addition
B. Subtraction
C. Division
D. Shift
✅ Answer: A

44. Shift-and-add algorithm uses:
A. Shifting and adding
B. Only adding
C. Only shifting
D. None
✅ Answer: A

45. Multiplication requires:
A. Registers
B. ALU
C. Control unit
D. All
✅ Answer: D

46. Binary multiplication is similar to:
A. Decimal
B. Octal
C. Hex
D. None
✅ Answer: A

## ⚡ Booth Multiplication (47–50)

47. Booth algorithm is used for:
A. Division
B. Multiplication
C. Addition
D. Subtraction
✅ Answer: B

48. Booth algorithm handles:
A. Positive numbers only
B. Negative numbers
C. Zero only
D. None
✅ Answer: B

49. Booth algorithm reduces:
A. Steps
B. Hardware
C. Power
D. Memory
✅ Answer: A

50. Booth algorithm uses:
A. Encoding
B. Decoding
C. Shifting
D. All
✅ Answer: D

### 1. What is pipelining in computer architecture?

A. Executing one instruction at a time
B. Executing multiple instructions in overlapping phases
C. Storing instructions in memory
D. None of the above
Answer: B

### 2. The main goal of pipelining is to:

A. Reduce instruction size
B. Increase throughput
C. Decrease memory usage
D. Reduce clock speed
Answer: B

### 3. Pipeline improves:

A. Latency
B. Throughput
C. Both A and B
D. None
Answer: B

### 4. Number of stages in a basic RISC pipeline:

A. 3
B. 4
C. 5
D. 6
Answer: C

### 5. Which is NOT a pipeline stage?

A. Fetch
B. Decode
C. Execute
D. Compile
Answer: D

### 6. Pipeline hazards are:

A. Errors in memory
B. Situations that prevent next instruction execution
C. Cache misses
D. None
Answer: B

### 7. Structural hazard occurs due to:

A. Data dependency
B. Resource conflict
C. Branching
D. Cache
Answer: B

### 8. Data hazard arises when:

A. Two instructions use same resource
B. Instructions depend on previous results
C. Branch occurs
D. None
Answer: B

### 9. Control hazard is caused by:

A. Data dependency
B. Branch instructions
C. Memory
D. ALU
Answer: B

### 10. Ideal speedup of pipeline is:

A. Equal to number of stages
B. Half of stages
C. Double of stages
D. Zero
Answer: A

### 11. Pipeline register is used to:

A. Store data temporarily between stages
B. Store final output
C. Increase memory
D. None
Answer: A

### 12. CPI in ideal pipeline is:

A. 0
B. 1
C. 2
D. Depends
Answer: B

### 13. Which hazard is solved using forwarding?

A. Structural
B. Data
C. Control
D. None
Answer: B

### 14. Pipeline stall is also called:

A. Bubble
B. Hazard
C. Cycle
D. Delay slot
Answer: A

### 15. Branch prediction helps in:

A. Reducing control hazards
B. Reducing memory
C. Increasing hazards
D. None
Answer: A

### 16. Superscalar architecture means:

A. Single instruction at a time
B. Multiple instructions per cycle
C. Slow processing
D. None
Answer: B

### 17. Pipeline efficiency is affected by:

A. Hazards
B. Stalls
C. Branching
D. All of the above
Answer: D

### 18. Data forwarding is also called:

A. Bypassing
B. Stalling
C. Branching
D. None
Answer: A

### 19. Which is first stage?

A. Decode
B. Execute
C. Fetch
D. Memory
Answer: C

### 20. Last stage is:

A. Decode
B. Write Back
C. Fetch
D. Execute
Answer: B

### 21. Throughput means:

A. Time per instruction
B. Instructions per unit time
C. Memory usage
D. None
Answer: B

### 22. Latency means:

A. Total time for one instruction
B. Throughput
C. Speedup
D. None
Answer: A

### 23. Pipeline is similar to:

A. Assembly line
B. Memory
C. Cache
D. Disk
Answer: A

### 24. Structural hazards can be avoided by:

A. Adding hardware resources
B. Reducing instructions
C. Branching
D. None
Answer: A

### 25. RAW hazard stands for:

A. Read After Write
B. Write After Read
C. Write After Write
D. None
Answer: A

### 26. WAR hazard stands for:

A. Read After Write
B. Write After Read
C. Write After Write
D. None
Answer: B

### 27. WAW hazard stands for:

A. Read After Write
B. Write After Read
C. Write After Write
D. None
Answer: C

### 28. Control hazards can be minimized using:

A. Branch prediction
B. Forwarding
C. Stalls
D. None
Answer: A

### 29. Pipeline depth refers to:

A. Number of instructions
B. Number of stages
C. Memory size
D. None
Answer: B

### 30. Pipeline clock cycle time is determined by:

A. Longest stage delay
B. Shortest stage delay
C. Average delay
D. None
Answer: A

### 31. Interlocking is used to:

A. Detect hazards
B. Remove hazards
C. Store data
D. None
Answer: A

### 32. Pipeline registers increase:

A. Delay
B. Speed
C. Memory
D. None
Answer: A

### 33. Speedup =

A. Non-pipelined time / pipelined time
B. Pipelined / non-pipelined
C. Sum
D. None
Answer: A

### 34. Pipeline works best when:

A. Instructions are independent
B. Dependent
C. Branch-heavy
D. None
Answer: A

### 35. Bubble reduces:

A. Throughput
B. Speed
C. Efficiency
D. All
Answer: D

### 36. Multiple pipelines mean:

A. Parallelism
B. Sequential execution
C. Slow speed
D. None
Answer: A

### 37. Control unit manages:

A. Pipeline flow
B. Memory
C. ALU
D. Cache
Answer: A

### 38. Forwarding unit resolves:

A. Structural hazards
B. Data hazards
C. Control hazards
D. None
Answer: B

### 39. Flushing pipeline occurs in:

A. Branch misprediction
B. Data hazard
C. Structural hazard
D. None
Answer: A

### 40. Pipeline hazard detection unit is used for:

A. Detect hazards
B. Execute instructions
C. Store data
D. None
Answer: A

### 41. Ideal pipeline has:

A. No hazards
B. Many hazards
C. Stalls
D. None
Answer: A

### 42. Which improves pipeline performance?

A. Instruction scheduling
B. Branching
C. Stalls
D. None
Answer: A

### 43. Instruction pipeline differs from arithmetic pipeline by:

A. Function
B. Purpose
C. Operation
D. All
Answer: D

### 44. Pipeline execution is:

A. Sequential
B. Parallel overlapping
C. Random
D. None
Answer: B

### 45. Pipeline control is:

A. Complex
B. Simple
C. None
D. Static
Answer: A

### 46. Pipeline hazards reduce:

A. Speed
B. Efficiency
C. Throughput
D. All
Answer: D

### 47. Instruction issue rate means:

A. Instructions per cycle
B. Memory
C. Delay
D. None
Answer: A

### 48. Deep pipeline means:

A. More stages
B. Less stages
C. No stages
D. None
Answer: A

### 49. Pipeline balancing ensures:

A. Equal stage delays
B. Unequal delays
C. Faster memory
D. None